tlp

5-LEVEL CASCADED H-BRIDGE INVERTER

Complete Build, Test & Assembly Guide

Controller-Agnostic Power Stage with Full Isolated Sensing

Version 3.1 — Final

ELE 401/402 Graduation Project

Hacettepe University — Electrical and Electronics Engineering

February 2026

Ahmet • Gökhan • Mücahit • Furkan

# 1. SYSTEM OVERVIEW

This document is the single, authoritative reference for building, wiring, and testing the 5-Level Cascaded H-Bridge (CHB) Inverter. Every resistor, capacitor, and connection is specified. It supersedes all prior BOM lists and wiring guides.

## 1.1 Architecture Summary

Two H-Bridge power modules are connected in series at their AC outputs. Each bridge is powered by an independent 50V DC source. The combined output produces a 5-level staircase waveform (+100V, +50V, 0V, -50V, -100V) at 50Hz with 5kHz PWM switching.

The power board is designed as a controller-agnostic module. Its interface consists of standard digital SPI and PWM signals. Any controller — STM32, FPGA, RISC-V SoC, or Arduino — can drive the board without hardware modification. Only the pull-up voltage on MISO return optocouplers needs to match the controller’s I/O voltage.

## 1.2 Key Specifications

| Parameter | Value |
| --- | --- |
| Topology | Single-Phase 5-Level Cascaded H-Bridge |
| DC Bus Voltage (per bridge) | 50V (independent isolated sources) |
| Peak AC Output Voltage | ±100V (2 × 50V) |
| Target Power Rating | ~700W into resistive load |
| Switching Frequency | 5 kHz |
| Modulation | In-Phase Disposition Level-Shifted PWM (IPD LS-PWM) |
| Controller | Any 3.3V/5V logic: STM32F303RE, FPGA, or custom RISC-V |
| Gate Driver | TLP250 Optically Isolated Driver (×8) |
| Gate Drive Power | B0515S-1W Isolated DC-DC, 2 paralleled per bridge |
| DC Bus Sensing | Isolated MCP3201 + 6N137 per bridge (full measurement) |
| Current Sensing | ACS712-20A on AC output, read via MCP3201 |
| Voltage Calculation | V_AC = V_DC1_measured × D1 + V_DC2_measured × D2 |

## 1.3 Key Design Features

- Controller-Agnostic Interface: Standardized digital header with SPI + PWM. Works with STM32, FPGA, or any microcontroller. No analog signals cross the interface.

- Full Isolated Sensing: Both DC buses are measured through galvanically isolated channels (MCP3201 + 6N137). System can handle real DC sources (batteries, solar panels) with voltage monitoring and protection.

- Real-Time Protection: Undervoltage, overvoltage, and overcurrent detection on all channels. Firmware/HDL can kill PWM outputs within one switching period.

- Symmetric Design: Both bridge sensing channels are identical. Same hardware, same firmware/HDL module instantiated twice.

## 1.4 Power Supply Requirements

| Supply | Voltage | Current | Purpose |
| --- | --- | --- | --- |
| Supply 1 | 50V DC | 0–15A | Bridge 1 power stage |
| Supply 2 | 50V DC | 0–15A | Bridge 2 power stage |
| Supply 3 | 5V DC, 2A | ~1.2A | B0515S modules + ACS712 + safe-side ADC |
| USB Cable | 5V USB | ~200mA | STM32 Nucleo (if used as controller) |

NOTE: Supply 3 can be replaced by an LM2596 buck converter module tapped from Bridge 1’s 50V bus. See Section 8.1.

TIP: For initial testing, an external 15V bench supply can bypass the B0515S modules entirely to test gate drivers independently. See Test Phase 2 in Section 6.

# 2. COMPLETE BILL OF MATERIALS

All quantities include recommended spares. Every component is listed with its exact role and circuit destination.

## 2A. Power Semiconductors

| Component | Qty (Need+Spare) | Role | Key Spec | Source |
| --- | --- | --- | --- | --- |
| IRFZ44N MOSFET | 8+4 = 12 | H-bridge switches | 55V, 49A, 17.5mΩ, TO-220 | Robotistan / Özdisan |
| TLP250 Optocoupler | 8+2 = 10 | Isolated gate drivers | 2500Vrms, 1.5A peak, DIP-8 | Özdisan |
| B0515S-1W DC-DC | 4+2 = 6 | Isolated 15V gate supply | 5V→15V, 1W, SIP-4 | Mouser / AliExpress |
| 1N5408 Diode | 2+2 = 4 | Reverse polarity protect. | 3A, 1000V, DO-201 | Direnc.net |
| UF4007 Fast Diode | 4+4 = 8 | Bootstrap charge diodes | 1A, 1000V, 75ns, DO-41 | Direnc.net |
| 1.5KE62A TVS Diode | 2+2 = 4 | Transient voltage clamp | 53V standoff, 84.5V clamp, DO-201 | Mouser / Farnell TR |

## 2B. Sensing Components

| Component | Qty (Need+Spare) | Role | Key Spec | Source |
| --- | --- | --- | --- | --- |
| ACS712-20A Module | 1+1 = 2 | AC output current sensor | 100mV/A, Hall-effect | Robotistan |
| MCP3201 ADC | 3+1 = 4 | 12-bit SPI ADC | DIP-8, single channel | Direnc.net / Mouser |
| 6N137 Optocoupler | 6+2 = 8 | High-speed SPI isolation | 10Mbit/s, DIP-8 | Direnc.net / Özdisan |
| 78L05 Regulator | 2+1 = 3 | Island 5V from 15V rail | 5V/100mA, TO-92 | Direnc.net |

NOTE: 3x MCP3201: 1 per bridge for DC bus sensing (isolated), 1 on safe side for current sensing (not isolated). All use the same SPI protocol — identical firmware/HDL driver.

## 2C. Power Passives (DC Bus, Per Bridge ×2)

| Component | Value | Qty Total | Role |
| --- | --- | --- | --- |
| Electrolytic Cap | 1000µF / 100V | 2 | DC link bulk storage (1 per bridge) |
| Ceramic Cap | 100nF / 50V | 2 | DC bus HF bypass (1 per bridge, near MOSFETs) |
| Bleeder Resistor | 100kΩ / 2W | 2 | Safety discharge (1 per bridge) |
| Blade Fuse + Holder | 15A | 2+2 spare | Series with each bridge DC(+) input |

## 2D. Gate Drive Passives (×8 Channels)

| Component | Value | Qty Total | Role |
| --- | --- | --- | --- |
| Resistor | 220Ω, 1/4W | 8 | TLP250 LED current limit (controller → TLP250) |
| Resistor | 22Ω, 1/4W | 8 | Gate series (TLP250 output → MOSFET gate) |
| Resistor | 10kΩ, 1/4W | 8 | Gate-source pull-down (keeps MOSFET OFF) |
| Ceramic Cap | 100nF / 25V | 8 | TLP250 VCC-GND bypass (Pin 8 to Pin 5) |

## 2E. Bootstrap Components (×4 High-Side Channels)

| Component | Value | Qty Total | Role |
| --- | --- | --- | --- |
| UF4007 Fast Diode | 1A, 1000V | 4 | Bootstrap charge (+15V → UF4007 → HS TLP250 VCC) |
| Ceramic/Electrolytic Cap | 10µF / 25V | 4 | Bootstrap energy storage (HS TLP250 Pin8 to Pin5) |

## 2F. Snubber Components (×8 MOSFETs)

| Component | Value | Qty Total | Role |
| --- | --- | --- | --- |
| Resistor | 22Ω, 2W | 8 | Snubber damping (series with cap, across D-S) |
| Film Capacitor | 2.2nF / 630V | 8 | Snubber cap (series with 22Ω, across D-S) |

## 2G. Isolated Supply Passives (Per Bridge ×2)

| Component | Value | Qty Total | Role |
| --- | --- | --- | --- |
| Resistor | 1Ω, 1/4W | 4 | B0515S current sharing (each output → 1Ω → rail) |
| Electrolytic Cap | 100µF / 25V | 2 | Combined B0515S output filter |
| Ceramic Cap | 100nF / 25V | 2 | HF bypass on 15V rail |
| Ceramic Cap | 100nF (input) | 4 | B0515S input bypass (1 per module) |

## 2H. DC Bus Sensing Passives (Per Island ×2)

| Component | Value | Qty Total | Role |
| --- | --- | --- | --- |
| Resistor | 100kΩ, 1%, metal film | 2 | Voltage divider upper leg |
| Resistor | 5.1kΩ, 1%, metal film | 2 | Voltage divider lower leg |
| Ceramic Cap | 100nF | 2 | Divider output filter |
| Resistor | 220Ω, 1/4W | 6 | 6N137 LED current limit (3 per island) |
| Resistor | 4.7kΩ, 1/4W | 6 | 6N137 output pull-ups (3 per island) |
| Ceramic Cap | 100nF | 8 | Bypass: MCP3201 (2), 6N137 (6) |
| Ceramic Cap | 1µF / 25V | 2 | 78L05 input filter (1 per island, ensures regulator stability) |
| Electrolytic Cap | 10µF / 16V | 2 | 78L05 output filter (1 per island) |

## 2I. Current Sensing Passives (Safe Side)

| Component | Value | Qty | Role |
| --- | --- | --- | --- |
| Resistor | 1kΩ, 1/4W | 1 | Divider upper (ACS712 OUT → 1k → ADC node) |
| Resistor | 1.5kΩ, 1/4W | 1 | Divider lower (ADC node → 1.5k → GND) |
| Ceramic Cap | 100nF | 1 | RC low-pass filter (ADC node to GND, fc ≈ 1.6kHz) |
| Ceramic Cap | 100nF | 1 | Safe-side MCP3201 VDD bypass |

## 2J. Connectors and Mechanical

| Component | Qty | Role |
| --- | --- | --- |
| Screw Terminal Block (2-pos, 15A+) | 4 | DC inputs (2 per bridge: + and -) |
| Screw Terminal Block (2-pos, 15A+) | 2 | AC output to load |
| Pin Header 2×10 (2.54mm) | 1 | Controller interface header |
| Heatsink (TO-220 clip-on) | 8 | MOSFET thermal management |
| 18 AWG silicone wire (red/black) | 2m each | Power wiring |
| Hookup wire (assorted) | 5m | Signal wiring |

## 2K. Grand Total Summary

| Category | Unique Parts | Total Pieces (incl. spares) |
| --- | --- | --- |
| MOSFETs (IRFZ44N) | 1 | 12 |
| Gate Driver ICs (TLP250) | 1 | 10 |
| Isolated DC-DC (B0515S-1W) | 1 | 6 |
| ADCs (MCP3201) | 1 | 4 |
| High-Speed Optocouplers (6N137) | 1 | 8 |
| Current Sensor (ACS712-20A) | 1 | 2 |
| Voltage Regulators (78L05) | 1 | 3 |
| Diodes (1N5408 + UF4007) | 2 | 12 |
| TVS Diodes (1.5KE62A) | 1 | 4 |
| Fuses (15A blade) | 1 | 4 |
| Electrolytic Capacitors (4 values) | 4 | 8 |
| Ceramic Capacitors (100nF + 10µF) | 2 | ~35 |
| Film Capacitors (2.2nF/630V) | 1 | 8 |
| Resistors (10 values) | 10 | ~56 |
| Connectors & Mechanical | ~5 types | ~15 |
| GRAND TOTAL | ~25 unique parts | ~195 pieces |

NOTE: Estimated total component cost: 900–1100 TL (including spares and international shipping for B0515S and TVS diodes).

# 3. DETAILED CONNECTIONS BY ZONE

The circuit is divided into six zones. Build and test each zone in order before proceeding.

## 3.1 ZONE 1: DC Input and Protection (Build 2 Identical Copies)

Handles the 50V DC input, protects against faults, and provides bulk energy storage.

### Schematic:

Terminal(+) → 15A Fuse → Node_A (main +50V bus). Terminal(-) → GND_HV (bridge floating ground).

Five components connect in parallel across Node_A and GND_HV:

- 1N5408: Cathode to Node_A, Anode to GND_HV. Reverse polarity protection. If DC cables are swapped, diode conducts and blows fuse.

- 1.5KE62A TVS Diode: Cathode to Node_A, Anode to GND_HV. During normal 50V operation, the TVS is inactive (standoff voltage 53V). If a spike exceeds ~62V, the TVS clamps it to 84.5V maximum — protecting the 55V-rated IRFZ44N MOSFETs. Unlike a MOV, the TVS has a sharp clamping knee and responds in nanoseconds.

- 1000µF / 100V Electrolytic: (+) to Node_A, (-) to GND_HV. Bulk energy reservoir for sharp MOSFET switching currents.

- 100nF Ceramic: Across Node_A and GND_HV, near MOSFETs. Handles HF noise the electrolytic is too slow to filter.

- 100kΩ / 2W Bleeder: Across Node_A and GND_HV. Drains cap after power-off. Discharge time: ~8 minutes to safe voltage.

WARNING: Bridge 2’s GND_HV is NOT system ground. It connects to the series output point. Never tie the two bridges’ GND_HV together.

### Zone 1 Parts Per Bridge:

| Component | Value | Qty |
| --- | --- | --- |
| Screw Terminal Block | 2-pos, 15A+ | 1 |
| Blade Fuse + Holder | 15A | 1 |
| 1N5408 Diode | 3A, 1000V | 1 |
| 1.5KE62A TVS Diode | 53V standoff, DO-201 | 1 |
| Electrolytic Cap | 1000µF / 100V | 1 |
| Ceramic Cap | 100nF / 50V+ | 1 |
| Resistor | 100kΩ / 2W | 1 |

## 3.2 ZONE 2: Isolated Gate Drive Power (Build 2 Identical Copies)

Two B0515S-1W modules are paralleled per bridge to create an isolated +15V rail with 2W capacity for the TLP250 gate drivers and the sensing island.

### Schematic:

INPUT SIDE (system ground referenced):

+5V_System ─┬─ B0515S_A Pin2 (+Vin)    100nF bypass across Pin1-Pin2

└─ B0515S_B Pin2 (+Vin)    100nF bypass across Pin1-Pin2

GND_System ─┬─ B0515S_A Pin1 (GND)

└─ B0515S_B Pin1 (GND)

OUTPUT SIDE (isolated, floating at bridge potential):

B0515S_A Pin4 (+Vout) ─ 1Ω ─┬─ +15V_Drive

B0515S_B Pin4 (+Vout) ─ 1Ω ─┘   |

├─ 100µF/25V electrolytic

├─ 100nF ceramic

B0515S_A Pin3 (GND_out) ─┬─ Drive_GND

B0515S_B Pin3 (GND_out) ─┘

Drive_GND connects to low-side MOSFET Sources in that bridge.

The 1Ω balancing resistors force current sharing between the two unregulated modules. At 50mA total load, voltage drop is only 25mV.

### Zone 2 Parts Per Bridge:

| Component | Value | Qty |
| --- | --- | --- |
| B0515S-1W | 5V → 15V isolated | 2 |
| Resistor | 1Ω, 1/4W | 2 |
| Electrolytic Cap | 100µF / 25V | 1 |
| Ceramic Cap | 100nF / 25V | 1 |
| Ceramic Cap | 100nF (input bypass) | 2 |

## 3.3 ZONE 3: H-Bridge Power Stage and Gate Drivers (Build 2 Copies)

### 3.3.1 MOSFET H-Bridge Topology (Per Bridge)

Node_A (+50V Bus)

|             |

+----+----+   +----+----+

| Q1 (HS) |   | Q3 (HS) |    HS = High-Side (bootstrap driven)

| Drain   |   | Drain   |

+----+----+   +----+----+

Node_X    |             |    Node_Y     (AC output nodes)

+----+----+   +----+----+

| Q2 (LS) |   | Q4 (LS) |    LS = Low-Side (direct driven)

| Drain   |   | Drain   |

+----+----+   +----+----+

|             |

GND_HV (Bridge DC-) = Drive_GND

AC output of each bridge = Node_X − Node_Y.

### 3.3.2 TLP250 Wiring (Per MOSFET, ×8 Total)

TLP250 Pinout:

| Pin | Name | Connection |
| --- | --- | --- |
| 1 | N/C | Leave unconnected |
| 2 | Anode (LED+) | Controller PWM pin → 220Ω → Pin 2 |
| 3 | Cathode (LED-) | GND_System (controller ground) |
| 4 | N/C | Leave unconnected |
| 5 | GND (driver side) | Ground reference (see Low-Side vs High-Side below) |
| 6 | Vout | → 22Ω → MOSFET Gate |
| 7 | Vout (inverted) | Tie to Pin 5 (GND) through 10kΩ for noise immunity (optional: leave open) |
| 8 | VCC | +15V supply (see Low-Side vs High-Side below) |

100nF ceramic bypass cap across Pin 8 and Pin 5 on every TLP250, placed as close to the IC as possible.

220Ω input resistor sets LED current: (3.3V − 1.2V) / 220Ω = 9.5mA. Within the 5–16mA reliable operating range.

Per MOSFET Gate Circuit:

- TLP250 Pin 6 → 22Ω → MOSFET Gate (limits peak current to ~0.68A)

- MOSFET Gate → 10kΩ → MOSFET Source (pull-down, ensures OFF during power-up)

- MOSFET Drain-Source: 22Ω 2W + 2.2nF/630V in series (snubber, damps turn-off ringing)

### 3.3.3 Low-Side TLP250 (Q2, Q4 per bridge — 4 total)

- Pin 8 (VCC) → +15V_Drive rail directly.

- Pin 5 (GND) → Drive_GND directly.

Simple: the low-side MOSFET source is always at Drive_GND.

### 3.3.4 High-Side TLP250 with Bootstrap (Q1, Q3 per bridge — 4 total)

UF4007 (fast diode)

+15V_Drive ── Anode ─►─ Cathode ─┬─ Node_Boot

|

10µF/25V cap (+)

|

High-Side MOSFET Source ──┼─ Node_VS (switching node)

|

10µF/25V cap (-)

TLP250 Pin 8 (VCC) ─ Node_Boot

TLP250 Pin 5 (GND) ─ Node_VS (= MOSFET Source)

100nF bypass cap across Pin 8 and Pin 5

How Bootstrap Works:

- Low-side ON: Node_VS pulled to Drive_GND (≈0V). UF4007 forward-biased. 10µF cap charges to ~14.3V (15V minus 0.7V diode drop).

- High-side ON: Node_VS rises to +50V. UF4007 reverse-biased (blocks). Cap provides floating 14.3V supply to TLP250.

- Cap discharges through TLP250 quiescent current (~16mA). At 95% duty cycle, on-time = 190µs. Voltage droop = (16mA × 190µs) / 10µF = 0.3V. Plenty of margin above TLP250’s 10V minimum.

- Next low-side ON period: cap recharges. Cycle repeats indefinitely.

WARNING: Maximum duty cycle: 95%. At 100%, the low-side never turns ON and the bootstrap cap never recharges. Enforce in firmware/HDL.

NOTE: Bootstrap pre-charge: At power-up, bootstrap caps are uncharged and high-side MOSFETs cannot switch. Firmware/HDL must include a 5–10ms pre-charge sequence at startup where only the low-side MOSFETs are pulsed (e.g., 50% duty cycle at 5kHz). This charges all bootstrap caps before beginning normal LS-PWM operation. Without pre-charge, the first few high-side switching attempts will produce weak or missing gate drive.

### Zone 3 Parts Per Bridge:

| Component | Value | Qty |
| --- | --- | --- |
| IRFZ44N MOSFET | 55V, 49A, TO-220 | 4 |
| TLP250 | Opto gate driver, DIP-8 | 4 |
| Resistor 220Ω | 1/4W | 4 |
| Resistor 22Ω | 1/4W (gate series) | 4 |
| Resistor 10kΩ | 1/4W (pull-down) | 4 |
| Resistor 22Ω | 2W (snubber) | 4 |
| Film Cap 2.2nF | 630V (snubber) | 4 |
| Ceramic Cap 100nF | 25V (TLP250 bypass) | 4 |
| UF4007 Diode | Fast recovery (bootstrap) | 2 (Q1, Q3 only) |
| Cap 10µF | 25V (bootstrap) | 2 (Q1, Q3 only) |
| Resistor 10kΩ | 1/4W (TLP250 Pin7 tie) | 4 (optional) |
| Heatsink TO-220 | Clip-on | 4 |

## 3.4 ZONE 4: Isolated DC Bus Sensing (Build 2 Identical Copies)

Each bridge has an isolated sensing island that measures the DC bus voltage and sends the digital data back to the controller through high-speed optocouplers. Both islands are identical in hardware, making the firmware/HDL symmetric.

### 3.4.1 Island Power Supply

The sensing island is powered from the existing +15V_Drive rail (Zone 2) through a 78L05 linear regulator:

+15V_Drive ── 1µF ceramic ─┬─ 78L05 Pin1 (IN) ─ 78L05 Pin3 (OUT) ─┬─ +5V_Island

|                                         |

|              78L05 Pin2 (GND)           ├─ 100nF ceramic

|                    |                    ├─ 10µF/16V electrolytic

Drive_GND ────────────┼────────────────── GND_Island

78L05 dissipation: (15V − 5V) × 25mA = 0.25W. TO-92 package handles this easily. This adds ~0.25W to the B0515S load, bringing total per bridge to ~0.75–1.25W, well within the 2W capacity of the paralleled B0515S modules.

### 3.4.2 Voltage Divider

Node_A (+50V) ── 100kΩ (1%) ── Sense_Pin ── 5.1kΩ (1%) ── GND_Island

|

100nF ceramic

|

GND_Island

|

MCP3201 Pin2 (IN+)

MCP3201 Pin3 (IN-) ─ GND_Island

Voltage at Sense_Pin: 50V × 5.1 / (100 + 5.1) = 2.42V. With 5V_Island as ADC reference, this uses 48% of the ADC range. At 60V overvoltage: 2.91V — safe. Resolution: 50V / (4096 × 0.0485) = 0.252V per count.

### 3.4.3 MCP3201 ADC Connections

| Pin | Name | Connection |
| --- | --- | --- |
| 1 | VDD/VREF | +5V_Island (also sets ADC reference voltage) |
| 2 | IN+ | Sense_Pin (divider output) |
| 3 | IN- | GND_Island |
| 4 | VSS | GND_Island |
| 5 | CLK | From 6N137 #1 output (isolated SCK) |
| 6 | DOUT | To 6N137 #3 input LED (isolated MISO return) |
| 7 | ̅C̅S̅/SHDN | From 6N137 #2 output (isolated CS) |
| 8 | VDD | Tie to Pin 1 (+5V_Island) |

100nF ceramic bypass cap across Pin 1 and Pin 4.

### 3.4.4 6N137 SPI Isolation (3 Per Island)

The 6N137 is an 8-pin high-speed optocoupler with a totem-pole output. Each has an enable pin and requires an output pull-up resistor.

6N137 #1 — SCK (Controller → Island):

CONTROLLER SIDE:                  ISLAND SIDE:

SCK pin ─ 220Ω ─ Pin2 (Anode)     Pin8 (VCC) ─ +5V_Island

GND_System ─── Pin3 (Cathode)   Pin7 (Enable) ─ GND_Island (always ON)

Pin5 (GND) ─ GND_Island

Pin6 (Vout) ─ 4.7kΩ to +5V_Island

└─ MCP3201 CLK (Pin5)

100nF bypass: Pin8 to Pin5

6N137 #2 — CS (Controller → Island):

Same wiring as #1, but controller CS pin drives the LED, and output goes to MCP3201 CS (Pin 7).

6N137 #3 — MISO/DOUT (Island → Controller):

ISLAND SIDE:                      CONTROLLER SIDE:

MCP3201 DOUT (Pin6)               Pin8 (VCC) ─ +3.3V (match controller I/O)

└─ 220Ω ─ Pin2 (Anode)         Pin7 (Enable) ─ GND_System (always ON)

GND_Island ─ Pin3 (Cathode)       Pin5 (GND) ─ GND_System

Pin6 (Vout) ─ 4.7kΩ to +3.3V

└─ Controller MISO pin

100nF bypass: Pin8 to Pin5

NOTE: For 6N137 #1 and #2: output-side VCC = +5V_Island. For 6N137 #3: output-side VCC = controller I/O voltage (3.3V for STM32/most FPGAs, 5V for Arduino). This is the ONLY thing that changes between controllers.

### Zone 4 Parts Per Island (x2 bridges):

| Component | Value | Qty Per Island |
| --- | --- | --- |
| 78L05 Regulator | 5V/100mA, TO-92 | 1 |
| MCP3201 ADC | 12-bit SPI, DIP-8 | 1 |
| 6N137 Optocoupler | High-speed, DIP-8 | 3 |
| Resistor 100kΩ | 1%, metal film | 1 |
| Resistor 5.1kΩ | 1%, metal film | 1 |
| Resistor 220Ω | 1/4W | 3 |
| Resistor 4.7kΩ | 1/4W | 3 |
| Ceramic Cap 100nF | Bypass + filter | 5 |
| Ceramic Cap 1µF | 78L05 input (stability) | 1 |
| Electrolytic Cap 10µF | 16V | 1 |

## 3.5 ZONE 5: Output Current Sensing (Safe Side)

The ACS712-20A Hall-effect sensor measures AC load current. Its output is conditioned and digitized by a dedicated MCP3201 on the safe (system ground) side. No isolation is needed because the ACS712 and this ADC share system ground.

### 3.5.1 ACS712 Connections:

- IP+ / IP-: In series with the AC output load wire (full load current flows through).

- VCC: +5V_System.

- GND: GND_System.

- OUT: Analog output. 2.5V at zero current, ±100mV per Amp.

### 3.5.2 Signal Conditioning and ADC:

ACS712 OUT ── 1kΩ ── ADC_Node ── 1.5kΩ ── GND_System

|

100nF ceramic (RC filter, fc ≈ 1.6kHz)

|

GND_System

ADC_Node ── MCP3201 Pin2 (IN+)

MCP3201 Pin3 (IN-) ─ GND_System

MCP3201 Pin1/8 (VDD) ─ +3.3V_System (or +5V_System)

MCP3201 Pin4 (VSS) ─ GND_System

100nF bypass across Pin1 and Pin4

MCP3201 SPI: CLK, CS, DOUT ─ directly to controller

(no optocouplers needed, same ground reference)

Signal Math (with 3.3V ADC reference):

- Divider ratio: 1.5k / (1k + 1.5k) = 0.6

- At 0A: 2.5V × 0.6 = 1.50V at ADC

- At 7A (rated): 3.2V × 0.6 = 1.92V at ADC

- At 20A (max): 4.5V × 0.6 = 2.70V at ADC

- All values safely below 3.3V. Resolution: ~0.013A per ADC count.

### Zone 5 Parts:

| Component | Value | Qty |
| --- | --- | --- |
| ACS712-20A Module | Hall-effect sensor | 1 |
| MCP3201 ADC | 12-bit SPI, DIP-8 | 1 |
| Resistor 1kΩ | 1/4W | 1 |
| Resistor 1.5kΩ | 1/4W | 1 |
| Ceramic Cap 100nF | RC filter + ADC bypass | 2 |

## 3.6 ZONE 6: Controller Interface Header

The controller connects to the power board through a single 2×10 pin header (2.54mm pitch). This is the ONLY connection between the controller and the power board.

### 3.6.1 Pin Assignment

| Pin | Signal | Dir. | Function | STM32 Pin / FPGA Pin |
| --- | --- | --- | --- | --- |
| 1 | PWM_1H | In | Bridge 1, Q1 (HS-Left) | PA8 (TIM1_CH1) / User-defined |
| 2 | PWM_1L | In | Bridge 1, Q2 (LS-Left) | PA7 (TIM1_CH1N) / User-defined |
| 3 | PWM_2H | In | Bridge 1, Q3 (HS-Right) | PA9 (TIM1_CH2) / User-defined |
| 4 | PWM_2L | In | Bridge 1, Q4 (LS-Right) | PA10 (TIM1_CH2N) / User-defined |
| 5 | PWM_3H | In | Bridge 2, Q1 (HS-Left) | PC6 (TIM8_CH1) / User-defined |
| 6 | PWM_3L | In | Bridge 2, Q2 (LS-Left) | PC7 (TIM8_CH1N) / User-defined |
| 7 | PWM_4H | In | Bridge 2, Q3 (HS-Right) | PC8 (TIM8_CH2) / User-defined |
| 8 | PWM_4L | In | Bridge 2, Q4 (LS-Right) | PC9 (TIM8_CH2N) / User-defined |
| 9 | SCK | In | Shared SPI clock for all 3 ADCs | SPI CLK pin |
| 10 | CS_DC1 | In | Chip select: Bridge 1 DC bus ADC | GPIO |
| 11 | CS_DC2 | In | Chip select: Bridge 2 DC bus ADC | GPIO |
| 12 | CS_CUR | In | Chip select: Current ADC | GPIO |
| 13 | MISO_DC1 | Out | SPI data from Bridge 1 DC ADC | SPI MISO (shared or separate) |
| 14 | MISO_DC2 | Out | SPI data from Bridge 2 DC ADC | SPI MISO (shared or separate) |
| 15 | MISO_CUR | Out | SPI data from Current ADC | SPI MISO (shared or separate) |
| 16 | FAULT_OUT | Out | Optional: active-low fault indicator | GPIO input (optional) |
| 17 | +5V In | Pwr | Powers B0515S, ACS712 | External 5V supply |
| 18 | +3.3V In | Pwr | Powers safe-side MCP3201, MISO pull-ups | Controller 3.3V or FPGA VCCO |
| 19 | GND | Pwr | System ground | GND |
| 20 | GND | Pwr | System ground (redundant) | GND |

### 3.6.2 SPI Bus Architecture

NOTE: SCK fan-out: Pin 9 drives 2× 6N137 LEDs (9.5mA each = 19mA) plus 1× MCP3201 input (~0). STM32 GPIOs can source 25mA — fine. For FPGAs with lower drive strength (<16mA), either increase the 6N137 SCK LED resistors from 220Ω to 330Ω (reduces current to 6.4mA each, still above 6N137’s 5mA minimum), or add a 74HC125 buffer IC on the SCK line.

All three MCP3201 ADCs share the same SCK line but have independent CS and MISO lines. The controller reads them sequentially:

Read sequence (one complete scan):

1. Assert CS_DC1 low, clock 16 bits on SCK, read MISO_DC1 → V_dc1

2. Assert CS_DC2 low, clock 16 bits on SCK, read MISO_DC2 → V_dc2

3. Assert CS_CUR low, clock 16 bits on SCK, read MISO_CUR → I_out

At 1 MHz SPI clock: 48 clock cycles total = 48µs per scan

At 5kHz switching: 200µs per period → 4+ complete scans per period

NOTE: The MCP3201 has no MOSI pin (single-channel, always converts). Only SCK, CS, and DOUT (MISO) are used. This simplifies wiring through the 6N137 optocouplers — only 3 optocouplers per isolated island instead of 4.

WARNING: SPI clock must not exceed 2MHz. The 6N137 optocouplers have ~75ns propagation delay per direction. Round-trip delay (SCK out + MISO back) is ~150ns. At 2MHz (250ns half-period), data arrives with 100ns margin. At 4MHz it would fail. 1MHz is the recommended safe operating speed.

TIP: To save FPGA pins, the 3 MISO lines can be shared onto one physical wire using tri-state behavior (MCP3201 DOUT goes high-Z when CS is high). However, MISO_DC1 and MISO_DC2 come through optocouplers with pull-up resistors, so they don’t go high-Z cleanly. Keeping them separate is recommended.

# 4. CASCADED SERIES CONNECTION

## 4.1 Physical Wiring

Bridge 1                         Bridge 2

+----[H-Bridge]----+            +----[H-Bridge]----+

|                  |            |                  |

Node_X1            Node_Y1 === Node_X2            Node_Y2

|            (series connection point)             |

|                                                  |

AC Out (+) ──── [ACS712] ──── [LOAD] ──── AC Out (-)

Bridge 2’s GND_HV (DC negative terminal) connects to Bridge 1’s output node (Node_Y1). This puts Bridge 2 electrically “on top of” Bridge 1’s output.

## 4.2 Switching States

| Level | Voltage | Bridge 1 | Bridge 2 |
| --- | --- | --- | --- |
| +2Vdc | +100V | Q1+Q4 ON (+50V) | Q1+Q4 ON (+50V) |
| +Vdc | +50V | Q1+Q4 ON (+50V) | Q1+Q3 ON (0V) |
| 0 | 0V | Q1+Q3 ON (0V) | Q1+Q3 ON (0V) |
| −Vdc | −50V | Q2+Q3 ON (−50V) | Q1+Q3 ON (0V) |
| −2Vdc | −100V | Q2+Q3 ON (−50V) | Q2+Q3 ON (−50V) |

# 5. PROTECTION SYSTEM

With full isolated sensing on both bridges plus output current measurement, the system can detect and respond to all major fault conditions:

| Fault | Detection Method | Response | Response Time |
| --- | --- | --- | --- |
| Short circuit | 15A fuse blows | Power physically cut | ~10ms (fuse) |
| Overcurrent (gradual) | ACS712 → MCP3201 → controller | Firmware/HDL kills all PWM | ~1ms |
| Shoot-through | Dead-time in PWM generation | Prevention (never occurs) | N/A |
| Bridge 1 undervoltage | MCP3201 via 6N137 (isolated) | Firmware/HDL kills all PWM | ~1ms |
| Bridge 2 undervoltage | MCP3201 via 6N137 (isolated) | Firmware/HDL kills all PWM | ~1ms |
| Overvoltage (either bus) | Same DC bus sensors | Firmware/HDL kills all PWM | ~1ms |
| Cable disconnection | Voltage drops to 0V on sensor | Firmware/HDL kills all PWM | ~1ms |
| Load imbalance | Compare V_dc1 vs V_dc2 | Adjust modulation or shutdown | Software decision |
| Reverse polarity | 1N5408 conducts + fuse blows | Power physically cut | ~10ms (fuse) |
| Voltage spike/transient | 1.5KE62A TVS diode clamps at 84.5V | Energy absorbed, MOSFET safe | ~ns |

No protection gaps. The system handles regulated bench supplies and real DC sources (batteries, solar panels) equally well.

## 5.1 Firmware/HDL Protection Logic

// Read all sensors (48µs at 1MHz SPI):

V_dc1 = read_mcp3201(CS_DC1) * (105.1 / 5.1) * (5.0 / 4096);

V_dc2 = read_mcp3201(CS_DC2) * (105.1 / 5.1) * (5.0 / 4096);

I_raw = read_mcp3201(CS_CUR) * (Vref / 4096);

I_out = (I_raw / 0.6 - 2.5) / 0.1;  // Undo divider, ACS712 formula

// Protection checks:

if (V_dc1 < 40.0 || V_dc2 < 40.0)    → FAULT_UNDERVOLTAGE

if (V_dc1 > 58.0 || V_dc2 > 58.0)    → FAULT_OVERVOLTAGE

if (fabs(I_out) > 15.0)               → FAULT_OVERCURRENT

if (fabs(V_dc1 - V_dc2) > 10.0)       → FAULT_IMBALANCE

// Sensorless output voltage using MEASURED bus values:

V_ac_out = V_dc1 * D_bridge1 + V_dc2 * D_bridge2;

# 6. TEST PROCEDURES (Follow This Order)

WARNING: Do NOT skip phases. Each test builds confidence for the next. Skipping ahead risks destroying components.

## Phase 1: Continuity Check (No Power)

Equipment: Multimeter (continuity/diode mode)

- Check each MOSFET Gate-Source: should be open circuit. Short = solder bridge.

- Check each MOSFET Drain-Source: open one way, ~0.5V diode drop the other (body diode).

- Check +50V bus (Node_A) to GND_HV: should NOT beep (not shorted).

- +15V_Drive to Drive_GND: should NOT beep.

- Verify 1N5408 orientation: banded end (cathode) faces Node_A.

- Verify UF4007 orientation: banded end (cathode) faces Node_Boot.

Pass: No unexpected shorts. All diodes correctly oriented.

## Phase 2: Gate Drive Test with External 15V (No 50V, No Load)

Equipment: 15V bench supply (200mA limit), oscilloscope, controller board

Purpose: Verify every TLP250 works before applying high voltage.

- Disconnect 50V supplies. Disconnect any load.

- Connect 15V supply directly to one bridge’s +15V_Drive and Drive_GND rails (bypassing B0515S).

- Power controller via USB. Generate 1kHz 50% duty cycle on one PWM channel at a time.

- Probe each MOSFET gate: should see clean 0V ↔ 14–15V square wave.

- Measure 15V supply current: ~10–20mA per active channel.

- Test complementary pairs (e.g., PWM_1H + PWM_1L together): verify dead-time gap on oscilloscope.

- Repeat for all 8 channels (both bridges, one at a time).

Pass: Clean gate waveforms. No overlap between complementary pairs. ~50–70mA per bridge.

WARNING: If waveforms ring or are sluggish, check the 100nF TLP250 bypass cap and 22Ω gate resistor. Bad bypassing is the #1 cause of noisy gate drive.

## Phase 3: B0515S Module Test

Equipment: 5V supply, multimeter

- Disconnect external 15V from Phase 2.

- Connect 5V to B0515S inputs (one bridge).

- Measure +15V_Drive output: should read 14.5–15.5V.

- Enable all 4 PWM channels for that bridge. Measure again: should still be >13V.

- 5V input current should be 100–200mA.

- Repeat for Bridge 2.

Pass: 13–16V under load. B0515S modules warm but not hot.

## Phase 4: Sensing Island Test (No 50V On Power Stage)

Equipment: 5V supply, controller, multimeter, oscilloscope (optional)

Purpose: Verify the MCP3201 + 6N137 isolated sensing chain works before applying high voltage.

- With 5V supply connected (B0515S modules running), verify +5V_Island output from 78L05: should read 5.0V ±0.2V.

- Apply a known low voltage (e.g., 5V from a bench supply) to one bridge’s DC input terminals. The voltage divider should produce: 5V × 5.1/105.1 = 0.243V at MCP3201 input.

- Run SPI read sequence from controller. MCP3201 should return: 0.243V / 5V × 4096 ≈ 199 counts.

- Scale up: apply 10V, 20V, 30V. Verify ADC readings scale linearly.

- Test the safe-side current ADC: with no load, ACS712 outputs 2.5V. After divider: 1.5V. MCP3201 (at 3.3V ref) should read: 1.5/3.3 × 4096 ≈ 1862 counts.

Pass: ADC readings match expected values within 2–3%. SPI communication reliable.

## Phase 5: Low-Voltage Power Test (10V Bus)

Equipment: 2×10V supplies (current limit 2A), oscilloscope, load (100Ω/25W or 60W light bulb)

- Connect 10V to each bridge’s DC input. Connect load through ACS712.

- Run LS-PWM at 5kHz/50Hz. Output should be a 5-level staircase at ±20V peak.

- Verify DC bus ADC readings match the 10V setting on your bench supplies.

- Verify current ADC shows sinusoidal variation around zero (~0.2A peak at 100Ω load).

- Run for 5 minutes. Check temperatures.

Pass: Clean staircase. ADC readings correct. No overheating. Stable for 5+ minutes.

## Phase 6: Full Voltage Test (50V Bus)

Equipment: 2×50V supplies (current limit: start at 2A), oscilloscope, rated load

- Increase voltage gradually: 10V → 20V → 30V → 40V → 50V. Monitor output waveform at each step.

- At 50V, increase current limit gradually: 2A → 5A → rated.

- Verify DC bus readings: both channels should show ~50V.

- Capture output waveform. Measure THD if oscilloscope supports FFT.

- Run for 15+ minutes at rated load. Log sensor data.

Pass: 5-level staircase at ±100V. THD < 5%. Sensors accurate. Thermally stable.

# 7. FIRMWARE / HDL NOTES

## 7.1 STM32 Timer Configuration

- TIM1 (Advanced) for Bridge 1, TIM8 (Advanced) for Bridge 2. Both support complementary outputs + dead-time.

- Timer clock: 72 MHz. ARR = 72M / 5k − 1 = 14399. Center-aligned mode.

- Dead-time: BDTR.DTG = 36–72 (500ns–1µs at 72MHz).

## 7.2 FPGA / RISC-V SoC Notes

- SPI master module: clock divider from system clock to ≤1MHz. 16-bit shift register per MCP3201.

- PWM module: 4 complementary channels with configurable dead-time. Directly instantiate two copies for two bridges.

- Protection logic can be purely combinational: if any sensor exceeds threshold, gate the PWM outputs to zero within one clock cycle. No software latency.

## 7.3 MCP3201 SPI Read Protocol

1. Pull CS low

2. Clock 16 bits on SCK (CPOL=0, CPHA=0)

3. DOUT returns: [null][B11][B10][B9][B8][B7][B6][B5][B4][B3][B2][B1][B0][X][X][X]

4. Pull CS high

5. Extract bits 1-12 from the 16-bit word: data = (raw >> 3) & 0x0FFF

## 7.4 Duty Cycle Clamping

WARNING: Never exceed 95% duty cycle. Enforce in firmware/HDL: if (CCR > 0.95 * ARR) CCR = 0.95 * ARR;

# 8. OPTIONAL IMPROVEMENTS

## 8.1 Derive 5V from 50V Bus (Fewer Bench Supplies)

An LM2596 adjustable buck module (~15–20 TL from Robotistan) can derive 5V from Bridge 1’s 50V bus:

Bridge 1 Node_A (+50V) ─ LM2596 VIN+

Bridge 1 GND_HV        ─ LM2596 VIN-

LM2596 VOUT+ ─ +5V_System

LM2596 VOUT- ─ GND_System

Only works from Bridge 1 (whose GND_HV is system ground). Adjust potentiometer to exactly 5.0V before connecting. Demo setup becomes: 2×50V supplies + USB cable only.

## 8.2 External 15V for Testing

During development, bypass B0515S modules by connecting 15V directly to +15V_Drive and Drive_GND. Disconnect B0515S outputs first (remove the 1Ω balancing resistors). See Test Phase 2.

## 8.3 Shared MISO Bus

If controller pins are scarce, the three MISO lines can be multiplexed onto one wire using a 3-to-1 analog mux (e.g., 74HC4053) controlled by the CS lines. The safe-side MCP3201 DOUT naturally goes high-Z when CS is high, but the isolated MISOs have pull-ups that prevent clean tri-stating. The mux solves this.

# 9. PCB LAYOUT GUIDELINES

## 9.1 High Current Tracks

- Terminal → Fuse → Node_A → MOSFET Drains: minimum 3mm track width. Expose copper (no solder mask) for manual solder reinforcement.

## 9.2 Isolation Gaps

- 2mm minimum clearance between high-voltage nets (50V, MOSFET D/S) and low-voltage control (5V, 3.3V).

- Only TLP250, B0515S, and 6N137 packages physically cross this gap.

## 9.3 Component Placement

- 100nF bypass caps within 5mm of every IC’s VDD pin.

- Bootstrap caps (10µF) close to their high-side TLP250.

- 1000µF bulk caps near MOSFET drain/source connections, not near input terminals.

- ACS712 as close to output terminals as possible.

- Sensing island components grouped together, separated from power stage by isolation gap.

- Controller header on board edge for easy cable access.

## 9.4 Ground Planes

- Do NOT use a single ground pour spanning the entire board.

- Each zone has its own local ground: GND_System, Drive_GND (per bridge), GND_Island (per bridge), GND_HV (per bridge).

- These grounds connect ONLY at their defined points. Mixing them creates ground loops and measurement errors.

— END OF DOCUMENT —